Oxy-Acetylene Safety Information Sheet

Oxygen

There is approximately _____% oxygen in normal air

Oxygen is an “____________”, it causes every thing it comes in contact with to burn hotter and faster.

If an oxygen bottle is leaking, the gas will settle in _____________before diffusing back into the atmosphere.

30% oxygen in the atmosphere will cause things to __________or__________ 8 times faster.

Dusting off clothing or work area with oxygen- the oxygen will remain present in the cloth fibers causing them to be more ____________________

An electric arc can cause an oxygen bottle to ____________(the carbon in the steel will atomize into a fuel)

Acetylene

Acetylene is a compound of ______________and__________ (C2H2) a member of the hydrocarbon gases.

It is an unstable gas, will violently decompose when in a pure state above _____ psi.

Has a burning temperature of 4,600o F, 5,700o F when burned with____________.

Auto-ignition temperature is 763o - 824o F, this means if acetylene reaches____ psi in a free state, it can explode by itself without a spark or flame being present.

Acetylene burns in air readily, and is most safely handled/stored in____________ filled with crushed firebrick wet with acetone.

Acetylene tanks must always be stored__________. This prevents the acetone and acetylene from separating.

Regulators and Hoses

Warning -Always keep the regulator free of_________, grease and other flammable substances

Never use oil or _____________on the regulator, cylinder or manifold connection

The hoses are usually color coded, but not always

_________(green)

__________ (red)

Be careful not to use other__________, such as air-lines, LP gas, etc.

Hoses are graded.

Make sure you are using the right _______for the right________.

Keep welding hoses clear of any failing_________,_________, or___________.

Never allow hoses to become coated with _________,__________or dirt.  Such coatings could conceal damaged areas.

_____________the hoses before attaching to welding torch handle or regulators.

If cuts, burns, worn areas or damaged fittings are found, ______________the hose.

Completely _______________welding hose if it contains multiple splices or when cracks or severe wear is noticed.

Never starve or choke a multi-flame heating nozzle.  This causes overheating of the head and a _________________may result.

Should a Flashback occur immediately turn off the _______________valve.

General Safety

Always work in a well-______________ area.

Always light the _______________first.

Open the _____________tank not more that 1.5 turns

Open the _____________tank all the way.

Use an approved__________, never matches or lighters.

Keep work area and clothes free of_______ or _________.

Keep work area free of all ______________materials.

Always wear proper________.

You are responsible for the ___________ you generate.