Oxy-Acetylene Safety Information Sheet

Oxygen

There is approximately 21% oxygen in normal air

Oxygen is an “accelerant”, it causes every thing it comes in contact with to burn hotter and faster.

If an oxygen bottle is leaking, the gas will settle in low areas before diffusing back into the atmosphere.

30% oxygen in the atmosphere will cause things to burn or explode 8 times faster.

Dusting off clothing or work area with oxygen- the oxygen will remain present in the cloth fibers causing them to be more flammable

An electric arc can cause an oxygen bottle to explode (the carbon in the steel will atomize into a fuel)

Acetylene

Acetylene is a compound of Hydrogen and Carbon (C2H2) a member of the hydrocarbon gases.

It is an unstable gas, will violently decompose when in a pure state above 15 psi.

Has a burning temperature of 4,600o F, 5,700o F when burned with oxygen.

Auto-ignition temperature is 763o - 824o F, this means if acetylene reaches 30 psi in a free state, it can explode by itself without a spark or flame being present.

Acetylene burns in air readily, and is most safely handled/stored in cylinders filled with crushed firebrick wet with acetone.

Acetylene tanks must always be stored upright. This prevents the acetone and acetylene from separating.

Regulators and Hoses

Warning -Always keep the regulator free of oil, grease and other flammable substances

Never use oil or grease on the regulator, cylinder or manifold connection

The hoses are usually color coded, but not always

Oxygen (green)

Acetylene (red)

Be careful not to use other hoses, such as air-lines, LP gas, etc.

Hoses are graded.

Make sure you are using the right hose for the right gas.

Keep welding hoses clear of any failing metal, slag or sparks.

Never allow hoses to become coated with oil, grease or dirt.  Such coatings could conceal damaged areas.

Examine the hoses before attaching to welding torch handle or regulators.

If cuts, burns, worn areas or damaged fittings are found, replace the hose.

Completely replace welding hose if it contains multiple splices or when cracks or severe wear is noticed.

Never starve or choke a multi-flame heating nozzle.  This causes overheating of the head and a flashback may result.

Should a Flashback occur immediately turn off the Oxygen valve.

General Safety

Always work in a well-ventilated area.

Always light the Acetylene first.

Open the Acetylene tank not more that 1.5 turns

Open the Oxygen tank all the way.

Use an approved striker, never matches or lighters.

Keep work area and clothes free of oil or grease.

Keep work area free of all flammable materials.

Always wear proper PPE.

You are responsible for the sparks you generate.