Jointer

JOINTER SAFETY:

1. Be sure that the Guard is in place on the Jointer, except when rabbeting.

2. The Jointer is set to remove a 1/16th of an inch each time you joint your board.

3. A Push Shoe should be used when the stock is below the top of the fence.

4. The proper technique of successfully jointing your boards, is to push down with your left hand as your board

passes onto the out-feed table and push forward with your right hand.

5. Boards should be at least 12 inches long for safe jointing.

6. The speed the stock is pushed over the cutter head should be slowly, while holding the stock firmly against the

table and fence.

7. Be especially careful of Loose Clothing, Jewelry and Long hair around the Jointer.

8. Be sure to turn on the woodshops ventilation system to remove the sawdust created by the Jointer and avoid clogging the jointer and creating an overflow on to the shop floor.

9. The purpose of the Jointer is to Level your stock to take out an Unevenness that may be in the board.

10.  If the Jointer motor lugs/slows down when you are machining your board, slow the speed of the jointing operation.

11. Do not Joint any stock that is less than 1/2" inch thick or less than 1 inch wide.

12. Planing your stock is not a substitute for Jointing your stock, the warp will not be removed from the stock.

Jointer safety rules:

1.  Keep your hands at least 4 inches from the blade. (safety margin)

2.  Use the guard at all times except rabbeting.

3.  Never cut more than 1/16 inch per pass.

4.  Do not joint stock less than 11 inches long.

5.  Use a push stick when jointing stock that is less than the height of the jointer fence.

6.  Do not edge joint wood that is thinner than 1 inch wide.

7.  Do not face joint wood that is thinner than ½ inch.

8.  Do not joint stock with loose knots, nails, or other foreign material

9.  Make sure that the stock stays in contact with both the fence and the table while cutting. Failure

to do this may cause the board to kick up and cause injury.

10.  Disconnect the power before changing or working on the knives.