Miter Saw

RADIAL ARM SAW & MITER/CHOP:

1.  Stock should be held firmly against the Fence and Table.

2. Boards shorter than 6 inches should not be cut on the radial arm saw.

3. Keep hands at least 4 inches from the saw blade and clear of the Saw track.

4. Never Cross your hands when making a cut on the radial arm saw.

5. The Radial Arm Saw is used to Crosscut across the grain of the Stock you are cutting.

6. Do not let the Saw Blade touch the stock when turning on the saw, this could cause a dangerous situation.

7.  Before cutting stock, be sure to Grasp the saw handle the entire time the saw blade is turning.

8. When cutting stock, feed the blade Slowly and Carefully into the board.

9. When you are finished cutting your stock , be sure to return the saw to the Rear of the arm.

10. Wait until the saw blade has completely Stopped before reaching in to remove any Scrap material.

11 When cutting on the Chop Saw, Lean into the cut as you cut, as the saw track gathers dust and gets sticky.

12. the Chop is used to make a Finished cuts when machining your work.

13.  Our Radial Arm Saw is used for make only Rough cuts when machining your work

Miter/Chop saw safety rules:

1.  Keep your hands at least 4 inches from blade. (Safety margin)

2.  Do not cross your hands when cutting.

3.  Make sure that the stock (board) is securely placed against the fence.

4.  Cut only one board at a time.

5.  Wait until the saw reaches full RPM before beginning your cut.

6.  Lower the saw slowly and evenly into the stock.

7.  Disconnect the power before changing the blade.